Micro-Cap 7

Installation:

This CD lets you install several versions of Micro-Cap 7:

• The Working Demo/Student version...does not require a security key.

• The Standard version...requires the white Hasp standard key.

• The LAN version...requires the red Hasp LAN key.

LAN installation

The LAN installation places MC7 on a network for use by a prescribed maximum number of simultaneous users. A single hardware key, resident on the server, mediates access by users.

User installation involves a single installation of MC7 on the server machine, and one or more installations on client machines. In addition, a network specific installation of device software on the server is required by the network administrator. The exact installation procedure is described in the file NETHASP.TXT.

New Features in Micro Cap 7

Optimizer

An optimizer was added for use in transient, AC, and DC analysis, providing the capability of minimizing, maximizing, or matching numeric values for any set of performance function expressions subject to specified constraints.

RF models and Smith charts

An S-Parameter two-port analog primitive was added to facilitate the modeling of RF devices. Polar plot and Smith chart plot styles were added to plot S-Parameter data.

Portable schematic files

Formerly, to send a schematic to a colleague, it was sometimes necessary to transfer not only the schematic file, but the component library file, the shape library file, and any model library files used. In Micro-Cap 7, the schematic file format was expanded to include all information necessary to draw and analyze the circuit. The necessary component, shape, and model library information is stored in the circuit file, so sending a circuit to another user requires only the circuit file.

User-specified paths (folders) for all major file groups

User-specified paths (folders) for circuits and libraries were added to allow easy separation of file types. Circuits can be stored in one folder and model libraries in another. Formerly, circuit files and libraries had to be in the current (last opened) folder or in the folder set by the MC6DATA environmental variable.

Characteristic curves

Many users have asked for a stimulus editor that would interactively show the waveform plot as edits are made. We liked the idea so much we added plots for most devices and incorporated it into the Attribute dialog box. Not just waveform sources, but active devices, diodes, Laplace sources, and digital stimulus sources all have characteristic curves or waveforms that can be plotted as you make edits. In the case of analog and digital waveform sources, the characteristic curve is the waveform itself. In the case of BJTs, one or more curves are selectable including DC current gain, saturation voltage, IV characteristics, and beta vs. frequency. Diodes and FETS have IV characteristics plots. OPAMPs have Bode plots.

Component editor improvements

The Component editor was significantly enhanced with an Import wizard that scans a model library file for subckts and adds them to the library when it finds existing parts with similar pin names.

This provides near-automatic addition of vendor libraries and makes their frequent updates easier to integrate into the libraries. Other improvements include an Add Part wizard, part templates for rapid manual entry of similar new parts, parts count by group, part name alphabetizer, and a part name list generator.

Attribute dialog box editor

The Attribute dialog box (that comes up when you double-click on a part) was changed to allow editing of model parameters, subckts, digital stimulus patterns, and analog waveforms from within the dialog box. This makes the ADB the place for all-in-one viewing, editing, and visualizing model-related data for components.

Multi-stage undo and redo

A multi-stage undo and redo was added to let you go back and forth one or more schematic steps.

Improved component find command

The component find command was expanded to search on the Name, Shape, Definition, and Memo fields.

A Beginning of Line option was added to simplify Memo field searches. A text output of the search results was provided to create custom lists of the library sections.

Text stepping

Stepping through a list of text labels was added to facilitate automated analysis of multiple pattern names, subckt files, model statements, and other text based material. For example, if XYZ is the model name for a BJT, then stepping XYZ through the list {2N2222A, 2N3906, 2N4114} produces three runs each using a unique set of model parameters for the transistor.

Symbolic derivative finder

MC7 introduced the symbolic derivative finder. It solves for and prints symbolic derivative formulas. For example, it will find the derivative of x^x (x raised to the power of x), with respect to x, as x*x^(x-1) + x^x*ln(x).

Status bar monitor

The status bar was expanded to show electrical information such as part name, type, current, and power as the mouse passes over the schematic parts. It also shows the step values for waveform branches in the analysis plot.

Waveform and data point labeling

Optional, automatic labeling of stepped waveform branches was added. Labeling of data points from a user-specified list was also added.

Multiple file opener

A capability to open more than one file at a time was added. MC7 now opens one or more files selected from the File dialog box.

Smooth schematic panning

Smooth 1 pixel panning was added. Earlier versions panned 8 pixels to accommodate earlier, slower machines.

Bill of materials

A bill of materials was added to the program. It lists the number, value, power, cost, and instances, for each electrical part type in the circuit.

Sanity checker

A model parameter range checker was added to warn when model parameters exceed user-specified "reasonable" limits.

SPICE file probing

Probing, the ability to click on an object and see its curve or waveform, was added to SPICE text files. This was actually added after the original MC6 release and was never in the MC6 manuals.

Analysis plot improvements

Several improvements were made to analysis plots, including the ability to switch between log and linear scales, to select the grid interval and the grid pattern, width, and thickness. Grid divisions can be conveniently specified round numbers and will remain stable during panning.

Shape editor improvements

Several improvements were added to the analysis plots, including the shape origin symbols, font size memory, and a drag copy command.

Live formulas

Live on-schematic text formulas were added that automatically update when any edits are made allowing live design formulas.

New X and Y Scale format

A new scale format, High, [Low], [Step], allows users to specify arbitrary grid intervals (scale divisions).

Even decimal values cursor positioning

A new cursor positioning mode was added for placing the cursor on even decimal values of the horizontal axis variable.

New thumbnail plot

A thumbnail plot was added to provide a global view of enlarged analysis plots, showing the location of the expanded region box superimposed on the whole plot.

Incremental auto-ranging

Y dimension auto-ranging was modified to apply only to the specified X range, not the whole simulation range.

Simultaneous multi-row analysis limits edits

Rapid analysis limits editing was facilitated by allowing users to change all of the rows in a waveform column simultaneously by clicking on the column header.

Run-invariant expressions in model parameters

Run-invariant expressions (those that don't change during a run) were added to model parameters. For example, a MOSFET VTO could be expressed as 2.0 + TEMP/100 to allow for threshold shift with temperature.

Complex trigonometric functions

All trigonometric and hyperbolic expressions were modified to accept and return complex arguments.

New trigonometric functions included cot, sec, csc, acot, asec, and acsc. New hyperbolic functions included coth, sech, csch, acoth, asech, and acsch.

Complex conditional functions

The MIN, MAX, and LIMIT functions acquired complex arguments.

Expressions for histograms and performance plots

In earlier versions of Micro-Cap, only individual performance functions were allowed. MC7 added individual performance function expressions as well. For example, expressions like Rise_Time(...)

+ Fall_Time(...) were allowed.

Attribute search and replace

A search and replace command was added to allow easy mass editing of model attributes.

PADS PCB output

The ability to write netlist files for interfacing to the PADS PCB program was added.